| VC_TBI | 09S | 09P | 10S | 10P | 11S | 11P | 12S | 12P |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Get an absolute dose .RD file from Eclipse |  |  |  |  |  |  |  |  |
| Make a folder for the simulation in tbi_clin and copy all of the codes into the new folder |  |  |  |  |  |  |  |  |
| Retrieve clinical egsphant and contourPoints files, and read data from the clinical egsphant |  |  |  |  |  |  |  |  |
| Python: read_and_scale_CPs_args.py |  |  |  |  |  |  |  |  |
| Matlab: mask_from_contours_AP/PA.m (Need to edit a few lines of the script) |  |  |  |  |  |  |  |  |
| C++ phantom creation: |  |  |  |  |  |  |  |  |
| ./puts_extend |  |  |  |  |  |  |  |  |
| ./shift_stack |  |  |  |  |  |  |  |  |
| Iterate the above step until the filter is centered over t0 |  |  |  |  |  |  |  |  |
| Remove duplicate materials from egsphant |  |  |  |  |  |  |  |  |
| Create egsinp for simulations (Need to edit Create_egsinp_ID.cc, then compile it and run the executable) |  |  |  |  |  |  |  |  |
| Move the final egsinp and egsphant to dosxyznrc |  |  |  |  |  |  |  |  |
| MC simulation - Make sure the .egsinp has the right path and filename for the .egsphant file |  |  |  |  |  |  |  |  |
| Clean up dosxyznrc, extract the .3ddose file |  |  |  |  |  |  |  |  |
| Calculate patient conversion factor in Excel |  |  |  |  |  |  |  |  |
| C++ dose processing: dose_strip_final.cc, move to MC |  |  |  |  |  |  |  |  |
| ./strip |  |  |  |  |  |  |  |  |
| ./sgolay |  |  |  |  |  |  |  |  |
| ./conv |  |  |  |  |  |  |  |  |
| Overwrite RD file (python Create_...USCD) |  |  |  |  |  |  |  |  |
| Bring dose files to MIM |  |  |  |  |  |  |  |  |
| For prone dose file: deal with x, y translation, make new RD files, iterate until fit properly |  |  |  |  |  |  |  |  |
| Deformable registration with doses |  |  |  |  |  |  |  |  |
| Contouring (on Supine image) and DVH analysis |  |  |  |  |  |  |  |  |